DMX Channels

| MODE 1 | MODE 2 | DMX Value | Function description |
| --- | --- | --- | --- |
| 1 | 1 | 0－255 | Pan |
| 2 | * | 0－255 | Pan Fine |
| 3 | 2 | 0－255 | Tilt |
| 4 | * | 0－255 | Tilt Fine |
| 5 | 3 | 0－255 | Pan/Tilt speed |
| 6 | 4 |  | Micro |
|  |  | 0－19 | Empty |
|  |  | 20－24 | Energy-saving mode（230W） |
|  |  | 25－129 | Empty |
|  |  | 130－139 | Lamp on by DMX |
|  |  | 140－149 | PAN/TILT reset |
|  |  | 150－159 | Empty |
|  |  | 160－169 | Small motors reset |
|  |  | 170－199 | Empty |
|  |  | 200－209 | Whole unit reset |
|  |  | 210－229 | empty |
|  |  | 230－239 | Lamp off by DMX |
|  |  | 240－255 | empty |
| 7 | 5 |  | Colors |
|  |  | 0－8 | White |
|  |  | 9－17 | Deep Red |
|  |  | 18－26 | Deep Blue |
|  |  | 27－35 | Yellow |
|  |  | 36－44 | green |
|  |  | 45－53 | Magenta |
|  |  | 54－62 | Light blue |
|  |  | 63－71 | red |
|  |  | 72－80 | Deep green |
|  |  | 81－89 | Amber |
|  |  | 90－98 | blue |
|  |  | 99－107 | orange |
|  |  | 108－116 | CTO |
|  |  | 117－127 | blue |
|  |  | 128－129 | white+blue |
|  |  | 130－134 | blue+CTO |
|  |  | 135－138 | CTO+orange |
|  |  | 139－143 | Orange+blue |
|  |  | 144－147 | blue+amber |
|  |  | 148－152 | Amber+deep green |
|  |  | 153－157 | Deep green+red |
|  |  | 158－161 | Red+light blue |
|  |  | 162－166 | Light blue+magenta |
|  |  | 167－171 | magenta+green |
|  |  | 172－176 | Green+yellow |
|  |  | 177－180 | Yellow+deep blue |
|  |  | 181－185 | Deep blue+deep red |
|  |  | 186－189 | Deep red+white |
|  |  | 190－220 | Run from fast to slow |
|  |  | 221－250 | Run from slow to fast |
|  |  | 251－255 | Color change when sound control |
| 8 | * |  | empty |
| 9 | 6 |  | empty |
|  | 7 |  | Static gobo |
| 10 |  | 0－3 | empty |
|  |  | 4－9 | GOBO1 |
|  |  | 10－15 | GOBO 2 |
|  |  | 16－21 | GOBO 3 |
|  |  | 22－27 | GOBO 4 |
|  |  | 28－33 | GOBO 5 |
|  |  | 34－39 | GOBO 6 |
|  |  | 40－45 | GOBO 7 |
|  |  | 46－51 | GOBO 8 |
|  |  | 52－57 | GOBO 9 |
|  |  | 58－63 | GOBO 10 |
|  |  | 64－69 | Small beam 1 |
|  |  | 70－75 | Small beam 2 |
|  |  | 76－87 | Small beam 3 |
|  |  |  | GOBO shaking from slow to fast |
|  |  | 88－95 | GOBO 1 |
|  |  | 96－103 | GOBO 2 |
|  |  | 104－111 | GOBO 3 |
|  |  | 112－119 | GOBO 4 |
|  |  | 120－127 | GOBO 5 |
|  |  | 128－135 | GOBO 6 |
|  |  | 136－143 | GOBO 7 |
|  |  | 144－151 | GOBO 8 |
|  |  | 152－159 | GOBO 9 |
|  |  | 160－167 | GOBO 10 |
|  |  | 168－175 | Small beam 1 |
|  |  | 176－183 | Small beam 2 |
|  |  | 184－199 | Small beam 3 |
|  |  | 200－255 | Static GOBO run from slow to fast |
| 11 | 8 |  | Rotate GOBO |
|  |  | 0－8 | empty |
|  |  | 9－17 | GOBO 1 |
|  |  | 18－26 | GOBO 2 |
|  |  | 27－35 | GOBO 3 |
|  |  | 36－44 | GOBO 4 |
|  |  | 45－53 | GOBO 5 |
|  |  | 54－62 | GOBO 6 |
|  |  | 63－71 | GOBO 7 |
|  |  | 72－80 | GOBO 8 |
|  |  | 81－89 | GOBO 9 |
|  |  |  | GOBO shaking from slow to fast |
|  |  | 90－97 | GOBO 1 |
|  |  | 98－106 | GOBO 2 |
|  |  | 107－115 | GOBO 3 |
|  |  | 116－124 | GOBO 4 |
|  |  | 125－133 | GOBO 5 |
|  |  | 134－142 | GOBO 6 |
|  |  | 143－151 | GOBO 7 |
|  |  | 152－160 | GOBO 8 |
|  |  | 161－169 | GOBO 9 |
|  |  | 171－179 | GOBO 9 not move |
|  |  | 180－217 | Run from fast to slow |
|  |  | 218－255 | Run from slow to fast |
| 12 | 9 |  | GOBO rotation |
|  |  | 0－127 | index |
|  |  | 128－177 | Run from slow to fast in anti clockwise |
|  |  | 178－203 | stop |
|  |  | 204－255 | Run from fast to slow in clockwise |
| 13 | * | 0－255 | empty |
| 14 | 10 |  | prism |
|  |  | 0－19 | empty |
|  |  | 20－75 | 6 linear prism |
|  |  | 76－255 | 8 facet prism |
| 15 | 11 |  | Prism rotation |
|  |  | 0－127 | index |
|  |  | 128－191 | Run from fast to slow in clockwise |
|  |  | 192－193 | empty |
|  |  | 194－255 | Run from slow to fast in anti clockwise |
| 16 | 12 |  | Frost |
|  |  | 0－64 | empty |
|  |  | 65－255 | empty |
| 17 | 13 |  | zoom |
|  |  | 0－255 | Linear zoom |
| 18 | * | 0－255 | Zoom fine |
| 19 | 14 | 0－255 | focus |
| 20 | * | 0－255 | Zoom fine |
| 21 | * | 0－255 | empty |
| 22 | 15 |  | strobe |
|  |  | 0－31 | Strobe close |
|  |  | 32－63 | Strobe open |
|  |  | 64－95 | Strobe from slow to fast |
|  |  | 96－127 | Strobe open |
|  |  | 128－143 | Fast to slow,slow close fast open |
|  |  | 144－159 | Slow to fast,fast open slow close |
|  |  | 160－191 | Strobe open |
|  |  | 192－223 | Random strobe from slow to fast |
|  |  | 224－255 | Strobe open |
| 23 | 16 |  | dimmer |
|  |  | 0－255 | Linear dimmer |
| 24 | * |  | empty |