DMX Channels

| 37 CH Mode | 41 CH Mode | Function | DMX Value | Function description |
| --- | --- | --- | --- | --- |
| 1 | 1 | Green+Blue | 0－255 | Linear Green+Blue |
| 2 | 2 | Magenta | 0－255 | Linear Magenta |
| 3 | 3 | Yellow | 0－255 | Linear Yellow |
| 4 | 4 | CTO | 0－255 | Linear CTO movement |
| 5 | 5 | Color wheel | 0-7 | white |
|  |  |  | 8-15 | white+colour1 |
|  |  |  | 16-23 | colour1 |
|  |  |  | 24-31 | colour1+colour2 |
|  |  |  | 32-39 | colour2 |
|  |  |  | 40-47 | Colour 2+colour 3 |
|  |  |  | 48-55 | colour3 |
|  |  |  | 56-63 | Colour 3+colour 4 |
|  |  |  | 64-71 | colour4 |
|  |  |  | 72-79 | Colour 4+ Colour5 |
|  |  |  | 80-87 | Colour5 |
|  |  |  | 88-95 | Colour5+Colour6 |
|  |  |  | 96-103 | Colour6 |
|  |  |  | 104-111 | Colour6+Colour7 |
|  |  |  | 112-119 | Colour7 |
|  |  |  | 120-127 | Colour7+white |
|  |  |  | 128-255 | Run from slow to fast |
| 6 | 6 | Strobe | 0－3 | Strobe Close |
|  |  |  | 4－103 | Strobe from slow to fast |
|  |  |  | 104－107 | Strobe Open |
|  |  |  | 108－207 | Random Strobe from slow to fast |
|  |  |  | 208－212 | Strobe Open |
|  |  |  | 213－251 | Random Strobe from slow to fast |
|  |  |  | 252－255 | Strobe Open |
| 7 | 7 | Dimming | 0-255 | Linear dimming |
| 8 | 8 | dimmer fine | 0-255 | No function |
| 9 | 9 | Aperture | 0-131 | Linear aperture from small to large |
|  |  |  | 132-171 | lris pulsation from slow to fast speed |
|  |  |  | 172-211 | lris pulsation from slow to fast speed with fast opening |
|  |  |  | 212-251 | lris pulsation from slow to fast speed with fast closing |
|  |  |  | 252-255 | Maximum aperture |
| 10 | 10 | Fixed GOBO | 0-7 | No function |
|  |  |  | 8-15 | GOBO 1 |
|  |  |  | 16-23 | GOBO 2 |
|  |  |  | 24-31 | GOBO 3 |
|  |  |  | 32-39 | GOBO 4 |
|  |  |  | 40-47 | GOBO 5 |
|  |  |  | 48-55 | GOBO 6 |
|  |  |  | 56-63 | GOBO 7 |
|  |  |  | 64-71 | GOBO 8 |
|  |  |  | 72-113 | Clockwise run from fast to slow. |
|  |  |  | 114-117 | Stop |
|  |  |  | 118-159 | Anti-clockwise run from fast to slow |
|  |  |  | 160-171 | GOBO 1 Shake |
|  |  |  | 172-183 | GOBO 2 Shake |
|  |  |  | 184-195 | GOBO 3 Shake |
|  |  |  | 196-207 | GOBO 4 Shake |
|  |  |  | 208-119 | GOBO 5 Shake |
|  |  |  | 220-231 | GOBO 6 Shake |
|  |  |  | 232-243 | GOBO 7 Shake |
|  |  |  | 244-255 | GOBO 8 Shake |
| 11 | 11 | Rotate GOBO | 0-18 | No function |
|  |  |  | 19-37 | GOBO 1 |
|  |  |  | 38-56 | GOBO 2 |
|  |  |  | 57-74 | GOBO 3 |
|  |  |  | 75-92 | GOBO 4 |
|  |  |  | 93-111 | GOBO 5 |
|  |  |  | 112-129 | GOBO 6 |
|  |  |  | 130-150 | GOBO 1 Shake |
|  |  |  | 151-171 | GOBO 2 Shake |
|  |  |  | 172-192 | GOBO 3 Shake |
|  |  |  | 193-213 | GOBO 4 Shake |
|  |  |  | 214-234 | GOBO 5 Shake |
|  |  |  | 235-255 | GOBO 6 Shake |
| 12 | 12 | GOBO Rota | 0-127 | Index |
|  |  |  | 128-190 | Rotation,Clockwise from fast to slow |
|  |  |  | 191-192 | Stop |
|  |  |  | 193-255 | Rotation,Anti-clockwise from slow to fast |
| 13 | 13 | GOBO Rota fine | 0-255 | No function |
| 14 | 14 | Prism | 0-127 | No function |
|  |  |  | 128-255 | Prism |
| 15 | 15 | Prism Rota | 0-127 | Index |
|  |  |  | 128-190 | Anti-clockwise run from fast to slow |
|  |  |  | 191-192 | Stop |
|  |  |  | 193-255 | Clockwise run from fast to slow |
| 16 | 16 | Frost | 0 | No function |
|  |  |  | 1-255 | Linear frost |
| 17 | 17 | Blade up 1 | 0-255 | Blade up 1 |
| 18 | 18 | Blade up 2 | 0-255 | Blade up 2 |
| 19 | 19 | Blade down 1 | 0-255 | Blade down 1 |
| 20 | 20 | Blade down 2 | 0-255 | Blade down 2 |
| 21 | 21 | Blade right 1 | 0-255 | Blade right 1 |
| 22 | 22 | Blade right 2 | 0-255 | Blade right 2 |
| 23 | 23 | Blade left 1 | 0-255 | Blade left 1 |
| 24 | 24 | Blade left 2 | 0-255 | Blade left 2 |
| 25 | 25 | Frame rotation | 0-255 | Frame rotation |
| 26 | 26 | Focus | 0－255 | Focus |
| 27 | 27 | Focus fine | 0－255 | Focus fine |
| 28 | 28 | Zoom | 0-255 | Linear zoom |
| 29 | 29 | Autofocus | 0－255 | No function |
| 30 | 30 | Autofocus fine | 0-255 | No function |
| 31 | 31 | Pan rotation | 0-255 | 0~540° |
| 32 | 32 | Pan Fine | 0-255 | Pan Fine |
| 33 | 33 | Tilt rotation | 0-255 | 0~270° |
| 34 | 34 | Tilt Fine | 0-255 | Tilt Fine |
| 35 | 35 | Tilt Fine | 0-255 | No function |
| 36 | 36 | Reset | 0-25 | No function |
|  |  |  | 26-76 | small step motor Reset |
|  |  |  | 77-127 | Pan/Tilt Reset |
|  |  |  | 128-255 | System reset |
| 37 | 37 | LED Lamp | 0-25 | No function |
| * | 38 | P/T speed | 0-255 | P/T ,Adjust speed |
| * | 39 | Color wheel speed | 0-255 | No function |
| * | 40 | Speed of light wheel | 0-255 | No function |
| * | 41 | GOBO wheel speed | 0-255 | No function |